ESP32 Water Check Controller V2.0

ระบบเช็คน้ำขาด by ช่างหนึ่ง

เอกสารคู่มือและซอร์สโค้ดสำหรับระบบควบคุมปั๊มน้ำด้วย ESP32, Flow Switch, Relay 2CH, LCD 16x2, NTP, MQTT และ Web Dashboard

# 1. System Overview

ตรวจจับสถานะ Flow Switch ในท่อน้ำ

เมื่อ Flow Switch หยุด ESP32 จะเริ่มนับถอยหลัง

ครบเวลาที่ตั้งไว้จะสั่ง Relay Pump ON อีกครั้ง

ตั้งค่าหน่วงเวลาและ MQTT ผ่าน Web Dashboard

แสดงวัน เวลา สถานะปั๊ม และ Countdown บน LCD 16x2

ใช้ NTP Server: time.nimt.or.th พร้อม fallback pool.ntp.org

# 2. Hardware

| Item | Qty | Note |
| --- | --- | --- |
| ESP32 DevKit V1 | 1 | Controller |
| Flow Switch | 1 | ตรวจจับการไหลของน้ำ |
| Relay Module 2CH | 1 | ควบคุมปั๊มและช่องสำรอง |
| LCD 16x2 I2C | 1 | แสดงผลภาคสนาม |
| 5VDC Power Supply | 1 | จ่ายไฟ ESP32/Relay ตามความเหมาะสม |
| Terminal Block/Fuse/Breaker | ตามจริง | ความปลอดภัยภาคสนาม |

## 2.1 Pin Map

| Function | GPIO | Note |
| --- | --- | --- |
| Flow Switch | GPIO34 | Digital input, input-only |
| Relay Pump | GPIO26 | Output |
| Relay AUX | GPIO27 | Output |
| LCD SDA | GPIO21 | I2C |
| LCD SCL | GPIO22 | I2C |
| Status LED | GPIO2 | Output |

# 3. Firmware Structure

ESP32_WaterCheck_v1/
├── ESP32_WaterCheck_v1.ino
├── config.h
├── pinmap.h
├── global.h
├── Storage.cpp/.h
├── FlowSwitch.cpp/.h
├── Relay.cpp/.h
├── LCDDisplay.cpp/.h
├── TimeSync.cpp/.h
├── MQTTClient.cpp/.h
├── Dashboard.cpp/.h
├── libraries.txt
└── data/

# 4. Main Logic

Flow ON  -> cancel countdown -> Pump ON
Flow OFF -> Pump OFF -> start countdown
Countdown complete -> Pump ON again -> continue monitoring
If flow returns before timeout -> countdown reset

# 5. Source Code Summary

config.h

pinmap.h

global.h

ESP32_WaterCheck_v1.ino

Storage.cpp/.h

FlowSwitch.cpp/.h

Relay.cpp/.h

LCDDisplay.cpp/.h

TimeSync.cpp/.h

MQTTClient.cpp/.h

Dashboard.cpp/.h

# 6. Default Dashboard

URL: http://<ESP32-IP>/

Default username: admin

Default password: 123456

Title: ระบบเช็คน้ำขาด by ช่างหนึ่ง

# 7. MQTT

| Direction | Topic | Payload |
| --- | --- | --- |
| Publish | watercheck/status | online/offline |
| Publish | watercheck/telemetry | JSON telemetry |
| Subscribe | watercheck/cmd | PUMP_ON, PUMP_OFF, AUX_ON, AUX_OFF, DELAY:300, RESTART |

# 8. Installation Steps

1. Install ESP32 board package in Arduino IDE

2. Install WiFiManager, PubSubClient, ArduinoJson, LiquidCrystal_I2C

3. Open ESP32_WaterCheck_v1.ino

4. Select ESP32 Dev Module

5. Upload firmware

6. Connect to WaterCheck_Setup AP

7. Enter WiFi credentials

8. Open ESP32 IP in browser

# 9. Safety Notes

งานไฟฟ้า 220VAC ต้องติดตั้งโดยช่างที่มีความรู้

ใช้ Fuse/Breaker/Contactor ให้เหมาะกับขนาดปั๊ม

Relay module ขนาดเล็กไม่ควรขับปั๊มกระแสสูงโดยตรง

ควรใช้กล่องกันน้ำและแยกส่วนไฟแรงดันต่ำ/สูง

ตรวจสอบ Common GND ระหว่าง ESP32 กับ Relay module

# 10. Files Included

| File | Description |
| --- | --- |
| Dashboard.cpp | Firmware source |
| Dashboard.h | Firmware source |
| ESP32_WaterCheck_v1.ino | Firmware source |
| FlowSwitch.cpp | Firmware source |
| FlowSwitch.h | Firmware source |
| LCDDisplay.cpp | Firmware source |
| LCDDisplay.h | Firmware source |
| MQTTClient.cpp | Firmware source |
| MQTTClient.h | Firmware source |
| Relay.cpp | Firmware source |
| Relay.h | Firmware source |
| Storage.cpp | Firmware source |
| Storage.h | Firmware source |
| TimeSync.cpp | Firmware source |
| TimeSync.h | Firmware source |
| config.h | Firmware source |
| global.h | Firmware source |
| libraries.txt | Firmware source |
| pinmap.h | Firmware source |
| TEST_PLAN.md | Documentation |
| WIRING.md | Documentation |